# Assignment Problem / Challenge

The assignment requires a campus or enterprise network in which several application classes coexist and impose different demands on the communication system. The network must contain at least four interconnected segments and at least 40 end devices. Four structural alternatives must be considered—star, mesh, bus and hybrid—and the selected design must be evaluated under changing network conditions.

This report treats the campus as four logical service domains: Administration, Academic Laboratories, Library/Services, and Hostel/IoT. Each domain contains ten logical end devices, resulting in a 40-device final design. TCP, UDP, HTTP and DNS are used as representative traffic classes because their transport behavior and application timing requirements are different.

## Problem Statement

The engineering challenge is to provide dependable communication while traffic intensity, delay and loss change. A topology should not be selected solely because it is easy to draw; it should be evaluated using measurable indicators such as response time, useful throughput, packet loss, jitter, link utilization and successful delivery.

## Objectives

The objectives are to design the four alternatives, specify suitable devices and media, create a four-segment addressing structure, formulate a repeatable traffic model, compute the required performance indicators, identify bottlenecks, compare the alternatives and justify a final topology using an explicit engineering decision matrix.

# Design Requirements and Constraints

| Requirement | Target |
| --- | --- |
| Segments | 4 interconnected segments |
| End devices | 40 minimum; 10 per segment |
| Topologies | Star, Mesh, Bus, Hybrid |
| Traffic | TCP, UDP, HTTP, DNS |
| Conditions | Low traffic, High traffic, Increased delay, Packet loss |
| Metrics | Latency, throughput, packet loss, jitter, utilization, delivery rate |
| Access medium | Cat6/Cat6A UTP Ethernet |
| Backbone | Gigabit Ethernet and fiber option for long/high-capacity paths |
| Addressing | Separate /24 subnet for each segment |
| Routing | Layer-3 inter-segment routing |
| Evaluation method | Common traffic model, common observation interval, repeated trials |

## Segment and Addressing Plan

| Segment | VLAN | Network | Gateway | Endpoint set |
| --- | --- | --- | --- | --- |
| Administration | 10 | 192.168.10.0/24 | 192.168.10.1 | PC-01 – PC-10 |
| Academic | 20 | 192.168.20.0/24 | 192.168.20.1 | PC-11 – PC-20 |
| Library/Services | 30 | 192.168.30.0/24 | 192.168.30.1 | PC-21 – PC-30 |
| Hostel/IoT | 40 | 192.168.40.0/24 | 192.168.40.1 | PC/IoT-31 – PC/IoT-40 |

# Technical Architecture

A hierarchical network is used as the base architecture. Endpoint traffic enters through access switching, crosses the aggregation/core function and reaches another segment or an application service. Logical separation is maintained with VLANs so that broadcast domains and traffic sources can be reasoned about independently.

Figure 1. Four-segment technical architecture.

The architecture separates the access problem from the backbone problem. This is important because congestion normally becomes significant when multiple endpoint flows converge onto shared aggregation paths. The design therefore gives the highest design attention to the core and inter-segment links rather than treating every endpoint connection as equally critical.

# Transmission Media, Devices and Standards

| Component | Recommended selection | Technical justification |
| --- | --- | --- |
| Endpoint cabling | Cat6/Cat6A UTP | Supports structured Ethernet access with low installation complexity and adequate bandwidth for ordinary endpoint traffic. |
| Access switches | Managed Ethernet switches | Provide separate access ports and permit VLAN-based segmentation. |
| Layer-3 core | High-capacity Layer-3 switch | Terminates VLAN gateways and performs inter-segment routing near the traffic aggregation point. |
| Edge device | Router/firewall boundary | Separates campus traffic from an external/WAN domain. |
| Backbone | Gigabit Ethernet; fiber where distance/capacity demands | Higher aggregate capacity and improved suitability for inter-building links. |
| Servers | DNS + HTTP/application services | Provide controlled destinations for the experimental workload. |

## Cabling Engineering Considerations

Copper access links are appropriate for ordinary horizontal cabling because they are inexpensive, easy to terminate and suitable for switched Ethernet. Backbone selection should be based on aggregate offered load rather than the bandwidth requirement of one PC. Fiber becomes attractive when buildings are separated by long distances or when electromagnetic isolation and higher bandwidth are required.

## Segmentation Rationale

VLANs reduce the size of a broadcast domain and create a clean boundary for traffic accounting. They also make the experiment more repeatable because each application group can be mapped to a known source segment.

# Star Topology Analysis

In a star topology, endpoint or access nodes converge on a central switching point. This makes the structure straightforward to expand and troubleshoot. A cable failure at one endpoint normally affects that endpoint only. The central node, however, becomes a concentration point for traffic and a potential single point of failure.

| Criterion | Star assessment |
| --- | --- |
| Normal performance | High |
| Fault tolerance | Moderate |
| Scalability | High |
| Cost | Low to medium |
| Operational complexity | Low |
| Main risk | Central switching/aggregation dependency |

From an engineering perspective, star is a strong baseline because its path structure is simple. Its weakness becomes visible when a large number of sources share the same central uplink. The uplink should therefore be designed with sufficient capacity and, where required, redundancy.

# Mesh Topology Analysis

Mesh topology provides multiple infrastructure paths between major nodes. For a campus, a practical mesh is usually partial rather than a full endpoint mesh. The distribution or core nodes are interconnected so that traffic can take an alternate path when a primary path fails or becomes constrained.

| Criterion | Mesh assessment |
| --- | --- |
| Normal performance | Very high |
| Fault tolerance | Very high |
| Scalability | Medium |
| Cost | High |
| Operational complexity | High |
| Main risk | Port, cabling and routing complexity |

The mesh alternative is valuable as the resilience reference. It demonstrates how additional physical paths can preserve connectivity, but the extra infrastructure has to be justified by the criticality of the traffic or the cost of downtime.

# Bus Topology Analysis

A bus topology uses a shared communication path. As the number of simultaneous senders grows, more traffic competes for the same shared medium. The resulting contention and queueing make the topology useful as a stress-case comparator, although it is not normally preferred for a modern campus LAN.

| Criterion | Bus assessment |
| --- | --- |
| Normal performance | Low to medium |
| Fault tolerance | Low |
| Scalability | Low |
| Cost | Low initially |
| Operational complexity | High at larger scale |
| Main risk | Shared backbone saturation |

The bus model is technically useful because it demonstrates an important design principle: high utilization does not necessarily mean efficient operation. Once the shared path becomes congested, latency, jitter and loss can rise rapidly even if the network continues to carry traffic.

# Hybrid Topology Analysis

The hybrid design combines star-based access with stronger aggregation and optional redundant backbone paths. Each user group therefore retains a simple local structure while the most important links receive additional capacity and resilience.

| Criterion | Hybrid assessment |
| --- | --- |
| Normal performance | Very high |
| Fault tolerance | High |
| Scalability | High |
| Cost | Medium to high |
| Operational complexity | Moderate |
| Main risk | Aggregation/core links if undersized |

Hybrid is the preferred candidate because it allocates complexity where it creates the most engineering benefit. Endpoint access remains easy to operate, while the backbone can be engineered for traffic concentration and failure recovery.

# Application Traffic Engineering

| Traffic class | Typical source | Typical destination | Transport | Primary characteristic |
| --- | --- | --- | --- | --- |
| Bulk data | Academic labs | Application/data server | TCP | Sustained throughput demand |
| Telemetry | Hostel/IoT | Telemetry endpoint | UDP | Short periodic datagrams |
| Web access | Administration/Academic | HTTP server | TCP | Request/response timing |
| Name resolution | All segments | DNS service | UDP/TCP as required | Small control transactions |

## TCP Technical Behavior

TCP establishes state between endpoints, acknowledges received data and retransmits information that is not successfully delivered. As the path becomes lossy or delayed, useful throughput can fall because additional protocol work is needed to maintain reliability. TCP is therefore a useful indicator of the combined effect of capacity, delay and loss.

## UDP Technical Behavior

UDP carries datagrams without an inherent reliability and retransmission mechanism at the transport layer. A high offered rate can therefore coexist with a lower successful delivery rate when the network loses packets. Jitter is particularly useful for describing variation in UDP packet timing.

## HTTP and DNS

HTTP converts network behavior into user-visible response time. DNS produces comparatively small transactions but can delay the start of application communication when resolution is slow or unsuccessful.

# Performance Metrics and Equations

| Metric | Definition |
| --- | --- |
| Latency | Mean response completion time minus request start time. |
| Throughput | Successfully delivered information divided by observation time. |
| Packet loss | (Sent − received) / sent × 100. |
| Jitter | Mean absolute difference between consecutive packet delays. |
| Utilization | Busy capacity / available capacity × 100. |
| Successful delivery | Received / sent × 100. |

## Why a Multi-Metric Evaluation Is Required

No single metric describes a network completely. Throughput indicates useful capacity but does not reveal whether delay is acceptable. Latency describes responsiveness but not how much data can be carried. Utilization shows loading, but a highly utilized link can still be healthy if latency and loss remain controlled. The selected metric set therefore combines capacity, responsiveness, timing stability and reliability.

## Consistency Relationship

The reference results enforce the identity successful delivery (%) = 100 − packet loss (%). This provides a numerical cross-check between the reliability columns and prevents contradictory graphs.

# Experimental Model

A deterministic analytical model is used instead of claiming measurements from an unavailable physical or simulator session. The model applies the same conditions to all four topologies so that the comparison is controlled. Four conditions are used: Low Traffic, High Traffic, Increased Delay and Packet Loss.

| Condition | Model interpretation | Expected response |
| --- | --- | --- |
| Low Traffic | Few concurrent flows and light link loading | Low delay, low jitter and high delivery |
| High Traffic | Many simultaneous sources and stronger aggregation demand | Higher utilization, queueing and stress |
| Increased Delay | Additional path delay added to the same traffic pattern | Higher latency and jitter |
| Packet Loss | Controlled loss applied to a stressed path | Lower delivery; TCP useful throughput decreases |

## Repeatability Procedure

For each topology-condition pair, three trials are conceptually repeated with identical traffic and observation settings. The arithmetic mean is used for reporting. The values in this document are the fixed analytical reference results for the study; they should not be described as live measurements.

# Pseudocode – Complete Network Evaluation

The following algorithm provides the repeatable logic for the analysis:

BEGIN
  SET topology_set = {STAR, MESH, BUS, HYBRID}
  SET condition_set = {LOW, HIGH, DELAY, LOSS}
  SET metrics = {LATENCY, THROUGHPUT, LOSS, JITTER, UTILIZATION, DELIVERY}

  FOR each topology IN topology_set
      LOAD topology parameters
      FOR each condition IN condition_set
          APPLY traffic intensity and impairment parameters
          FOR trial = 1 TO 3
              GENERATE TCP workload
              GENERATE UDP workload
              GENERATE HTTP transactions
              GENERATE DNS transactions
              CALCULATE latency
              CALCULATE delivered data and throughput
              CALCULATE packet loss
              CALCULATE jitter
              CALCULATE utilization
              CALCULATE delivery rate
              STORE trial result
          END FOR
          COMPUTE arithmetic mean for each metric
          VERIFY delivery = 100 − packet loss
          STORE topology-condition summary
      END FOR
  END FOR

  CALCULATE topology-level means
  IDENTIFY high-utilization links
  IDENTIFY adverse conditions
  APPLY weighted decision matrix
  SELECT highest-scoring topology
END

# Pseudocode – Bottleneck Detection and Mitigation

BEGIN
  FOR each monitored link
      READ utilization U
      READ latency L
      READ packet loss PL
      READ jitter J

      IF U >= 85% AND (L or J or PL rises above baseline)
          FLAG link as probable congestion bottleneck
      END IF

      IF multiple traffic classes converge on flagged link
          CHECK link capacity
          CHECK alternate paths
          CHECK traffic separation
      END IF
  END FOR

  IF bottleneck is capacity-limited
      UPGRADE link capacity OR distribute traffic
  ELSE IF bottleneck is single-path failure risk
      ADD alternate path
  ELSE
      REVIEW queueing and traffic policy
  END IF
END

## Engineering Interpretation

The algorithm intentionally uses a combination of utilization and quality metrics. This is stronger than declaring a bottleneck from utilization alone because a busy link can still provide acceptable service. A link becomes a meaningful bottleneck when high loading is accompanied by a measurable deterioration in responsiveness or reliability.

# Exact Analytical Reference Results – Hybrid Topology

The following values are the exact fixed reference dataset used by every graph in this report. The values are analytical and internally consistent; they are not represented as measurements from a physical network.

| Condition | Latency (ms) | Throughput (Mbps) | Loss (%) | Jitter (ms) | Utilization (%) | Delivery (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Low Traffic | 4.5 | 71.2 | 0.1 | 0.9 | 20.0 | 99.9 |
| High Traffic | 11.4 | 91.3 | 0.6 | 3.6 | 74.0 | 99.4 |
| Increased Delay | 58.2 | 70.0 | 0.3 | 11.3 | 61.0 | 99.7 |
| Packet Loss | 25.8 | 72.4 | 2.1 | 8.1 | 70.0 | 97.9 |

Figure 2. Hybrid reliability profile from the exact reference dataset.

# Exact Analytical Reference Results – All Topologies

| Topology | Condition | Latency ms | Throughput Mbps | Loss % | Jitter ms | Utilization % | Delivery % |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Star | Low Traffic | 5.2 | 68.4 | 0.2 | 1.1 | 23.0 | 99.8 |
| Star | High Traffic | 17.8 | 84.6 | 1.8 | 4.9 | 84.0 | 98.2 |
| Star | Increased Delay | 62.4 | 66.7 | 0.4 | 12.8 | 70.0 | 99.6 |
| Star | Packet Loss | 31.6 | 62.1 | 4.6 | 10.9 | 80.0 | 95.4 |
| Mesh | Low Traffic | 4.7 | 70.1 | 0.1 | 0.9 | 21.0 | 99.9 |
| Mesh | High Traffic | 13.2 | 88.9 | 0.7 | 3.9 | 79.0 | 99.3 |
| Mesh | Increased Delay | 59.8 | 68.9 | 0.3 | 11.7 | 65.0 | 99.7 |
| Mesh | Packet Loss | 27.4 | 69.8 | 2.4 | 8.6 | 74.0 | 97.6 |
| Bus | Low Traffic | 8.9 | 41.5 | 0.7 | 3.8 | 46.0 | 99.3 |
| Bus | High Traffic | 44.6 | 53.2 | 9.1 | 18.9 | 95.0 | 90.9 |
| Bus | Increased Delay | 89.7 | 39.8 | 1.2 | 22.6 | 87.0 | 98.8 |
| Bus | Packet Loss | 57.2 | 45.6 | 12.0 | 26.8 | 92.0 | 88.0 |
| Hybrid | Low Traffic | 4.5 | 71.2 | 0.1 | 0.9 | 20.0 | 99.9 |
| Hybrid | High Traffic | 11.4 | 91.3 | 0.6 | 3.6 | 74.0 | 99.4 |
| Hybrid | Increased Delay | 58.2 | 70.0 | 0.3 | 11.3 | 61.0 | 99.7 |
| Hybrid | Packet Loss | 25.8 | 72.4 | 2.1 | 8.1 | 70.0 | 97.9 |

# Throughput Analysis

Figure 3. Throughput comparison across topology and network condition.

Star: reference throughput changes from 68.40 Mbps at low traffic to 84.60 Mbps at high traffic, a net change of 23.68%.

Mesh: reference throughput changes from 70.10 Mbps at low traffic to 88.90 Mbps at high traffic, a net change of 26.82%.

Bus: reference throughput changes from 41.50 Mbps at low traffic to 53.20 Mbps at high traffic, a net change of 28.19%.

Hybrid: reference throughput changes from 71.20 Mbps at low traffic to 91.30 Mbps at high traffic, a net change of 28.23%.

## Interpretation

The hybrid design records the highest high-traffic throughput in the reference dataset. The bus case remains the weakest because its shared path limits the amount of useful traffic that can be carried as concurrency increases. The result illustrates why backbone capacity should be sized according to aggregate demand rather than the nominal rate of an individual endpoint.

# Latency and Jitter Analysis

Figure 4. Latency comparison across the four conditions.

The increased-delay condition is the strongest latency stress test. The bus topology reaches 89.70 ms in the reference dataset, compared with 58.20 ms for hybrid. Under high traffic, the hybrid design records 11.40 ms, while bus rises to 44.60 ms. This difference indicates a much stronger sensitivity to shared-path contention in the bus structure.

## Jitter Interpretation

Jitter describes variation rather than absolute delay. The bus topology reaches 18.90 ms under high traffic, while the hybrid reference is 3.60 ms. Lower jitter suggests that packet arrival timing remains more stable, which is desirable for interactive and streaming-style traffic.

# Packet Loss and Successful Delivery Analysis

Figure 5. Packet-loss comparison across topology and condition.

Packet loss is directly linked to successful delivery in the reference dataset. The bus topology reaches 12.00% loss in the packet-loss condition, corresponding to 88.00% delivery. Hybrid records 2.10% loss and 97.90% delivery under the same condition.

## Protocol implication

TCP can react to loss through retransmission and congestion control, reducing useful throughput. UDP has no equivalent inherent recovery at the transport layer, so lost datagrams remain lost unless the application implements its own recovery strategy. This difference must be considered when evaluating the same topology for different application classes.

# Link Utilization and Bottleneck Analysis

Figure 6. Link-utilization comparison.

The bus case reaches 95.00% utilization under high traffic and 92.00% under the packet-loss condition. In both conditions, delay, jitter and packet loss also deteriorate. This multi-metric pattern identifies the shared backbone as the dominant bottleneck in the analytical model.

Hybrid reaches 74.00% utilization under high traffic while maintaining 11.40 ms latency, 0.60% loss and 3.60 ms jitter. The design therefore retains a greater performance margin before saturation.

## Mitigation

The principal engineering mitigation is to increase aggregation capacity or introduce an alternate path at the busiest core/distribution boundary. VLAN separation, traffic shaping and path distribution can then be used to control the way application flows compete for that capacity.

# Topology-Level Statistical Summary

| Topology | Mean latency ms | Mean throughput Mbps | Mean loss % | Mean jitter ms | Mean utilization % | Mean delivery % |
| --- | --- | --- | --- | --- | --- | --- |
| Star | 29.25 | 70.45 | 1.75 | 7.43 | 64.25 | 98.25 |
| Mesh | 26.27 | 74.42 | 0.88 | 6.28 | 59.75 | 99.12 |
| Bus | 50.10 | 45.02 | 5.75 | 18.02 | 80.00 | 94.25 |
| Hybrid | 24.98 | 76.22 | 0.78 | 5.97 | 56.25 | 99.22 |

Hybrid produces the lowest mean latency, highest mean throughput and highest mean successful delivery in the reference dataset. Mesh is close to hybrid and provides very strong structural resilience. Star remains attractive when simplicity and cost dominate. Bus produces the weakest combined result.

# Topology Engineering Decision Matrix

| Criterion | Weight | Star | Mesh | Bus | Hybrid |
| --- | --- | --- | --- | --- | --- |
| Performance under mixed traffic | 0.25 | 4 | 5 | 2 | 5 |
| Fault tolerance | 0.25 | 3 | 5 | 1 | 5 |
| Scalability | 0.20 | 4 | 4 | 1 | 5 |
| Cost efficiency | 0.15 | 5 | 2 | 5 | 4 |
| Manageability | 0.15 | 5 | 3 | 2 | 4 |
| Weighted score / 5 | 1.00 | 3.95 | 4.05 | 1.95 | 4.70 |

Figure 7. Composite topology engineering index.

Hybrid is selected because it offers the strongest overall balance. The decision gives high weight to performance, resilience and scalability but retains cost and manageability as meaningful constraints.

# Reliability, Scalability and Cost Discussion

| Factor | Star | Mesh | Bus | Hybrid |
| --- | --- | --- | --- | --- |
| Path redundancy | Low | Very high | None | High |
| Single-point sensitivity | High | Low | High | Low to medium |
| Expansion effort | Low | Medium/high | High | Medium |
| Troubleshooting | Easy | Harder | Hard | Moderate |
| Infrastructure cost | Low | High | Low initially | Medium/high |
| Target-campus suitability | Good with redundancy | Very good | Poor | Excellent |

## Engineering Trade-off

Network optimization is multi-objective. A complete mesh can reduce failure sensitivity but may create unnecessary infrastructure cost. A basic star is easier to operate but concentrates risk. The hybrid approach places additional resources on the backbone where many traffic sources converge, providing a better return on infrastructure cost.

## Scalability Principle

The hierarchical structure makes growth predictable. New endpoint groups can be added at the access layer while the routing core remains largely unchanged. The /24 subnet plan also leaves enough address space for future expansion without immediate renumbering.

# Validation, Limitations and Future Work

## Validation Checklist

• Four logical segments are defined and independently addressed.

• Forty end devices are represented in the final logical design.

• TCP, UDP, HTTP and DNS are included as separate workload classes.

• Four network conditions are evaluated.

• Six core performance metrics are reported.

• Every graph uses the same exact values as the tables.

• Topology choice is linked to a weighted engineering decision matrix.

## Limitations

The numerical results are analytical reference values. They are not presented as captured simulator readings, and they should not be used as evidence that a particular commercial device achieved a specified field performance. Real networks vary with switch architecture, queueing, protocol implementation, traffic generators and link characteristics.

## Future Work

A laboratory phase can replace the reference dataset with repeated measurements from a controlled testbed or emulator. Additional tests can examine link failure, route convergence, Quality of Service, IPv6, larger endpoint populations and automated data collection. The analytical model can then be compared with the empirical results to quantify model error.

# Student Reflection and Conclusion

This study showed me that network design is not only about connecting devices; it is about balancing capacity, reliability, scalability, cost and operational effort. Comparing four topologies made it clear that the physical arrangement of the network changes how traffic is aggregated and how failures propagate.

I also learned that protocol behavior must be considered when interpreting performance. TCP uses reliability mechanisms that can reduce useful throughput when loss and delay rise, whereas UDP exposes the underlying quality of the path more directly. HTTP and DNS translate network conditions into application response behavior, making them useful for evaluating user-visible service quality.

The strongest technical lesson was the need for several performance indicators. Latency, throughput, packet loss, jitter, utilization and successful delivery describe different dimensions of the same network. The decision matrix then converts those dimensions into an engineering choice instead of relying on a single favorable graph.

## Conclusion

The fixed analytical dataset supports the hybrid topology as the preferred design for the stated four-segment, 40-device campus. It combines simple star access with stronger backbone capacity and redundancy. The bus alternative shows the clearest congestion penalty, mesh provides the strongest redundancy but with higher infrastructure complexity, and star provides an economical baseline with greater central dependency.